ANNEX 3a	-	MINIMUM REQUIREMENTS FOR THE

SUPPLIES AND THE TENDER

Tender form, please complete and return with the tender

Name of the tenderer:

INSTRUCTIONS

- The tenderer is asked to answer the following questions and to attach the required documents to its tender

- The tenderer shall answer all these questions concerning minimum requirements with an unqualified “Yes”, or otherwise, the Contracting authority must exclude the tenderer from the competitive bidding process.

- Please note, that the tenderer is requested to submit certain documents with the Tender or upon later request.

- All prices include all of the minimum requirements for the procurement of supplies listed in this form.

- The form may be completed in electronic format or by hand, but the contents of the form may not be altered.

| 1. MINIMUM REQUIREMENTS FOR THE PROCUREMENT |  |
| --- | --- |
| MINIMUM REQUIREMENT | TENDERER’S ANSWERS AND DOCUMENTS |
| Coils |  |
| Pancakes: The offer includes 17 double wound, double pancakes: | Yes No |
| Quality and dimensions of the Copper conductor: Oxygen free, high conductivity copper is used (conductivity at 20°C ≥ 5.8 x 107(Ωm)-1. Drawing IQU_ISI PC1.161.001_pancake 1 shows the dimensions of the conductor. Dimensions are met. If optional conductor size is used it has to meet the requirements presented in the technical specification and has been accepted by Customer. | Yes No |
| Dimension of the double wound double pancake: The dimensions (thickness, inner/outer radius), tolerances and number of turns (2 x 20 turns) are presented in Drawing IQU_ISI PC1.161.001_pancake 1. These requirements are met. | Yes No |
| Assembly of pancakes: Pancakes has to be divided into 3 sets: Solenoid 1 (7), Solenoid 2 (3) and Solenoid 3 (7). Number of pancakes inside the coil presented in the parenthesis. The offer includes the assembly of the pancakes in the following way: - Solenoid 1 and Solenoid 3: The pancakes (7) are assembled inside the low carbon steel yoke to form complete Solenoid 1 and Solenoid 3 (See: IQU_ISI PC1.161.000_solenoid 1 and IQU_ISI PC1.163.000_solenoid 3). The maximum dimensions reserved for pancakes are not exceeded. - Parts IQU_ISI PC1.161.005_yoke part 3 (in Solenoid 1) and IQU_ISI PC1.161.006_yoke part 4 (in Solenoid 3) are removable for dimensional inspection and machining (if later Bgrad optimization is needed). - The movement of pancakes inside the yoke structure is prevented (epoxy can be used to prevent the moving). - Solenoid 2: this solenoid includes 3 pancakes. Does not include yoke structure. Radial dimensions are same as for Solenoid 1 and Solenoid 3. The height of 3 pancakes, including the encasing epoxy, (forming Solenoid 2) is not allowed to exceed 77 mm, when assembled on a flat base (IQU_ISI PC1.162.000_solenoid 2). | Yes No Yes No Yes No Yes No |
| Manufactured low carbon steel parts: The dimensions and tolerances of each part has been presented in the drawings. The offer has to include manufacture of the following components: |  |
| IQU_ISI PC1.161.003_yoke part 1: Minimum requirement for steel: 1.97 T at 24 kA/m. | Yes No |
| IQU_ISI PC1.161.004_yoke part 2 (2 pieces): Minimum requirement for steel: 1.97 T at 24 kA/m. NOTE: Can be manufactured from 2 pieces instead of 1 piece (176 mm thickness => 2 x 88 mm thickness). In this case proper centering of the pieces has to be taken care of. See drawing for maximum removed volume for feedthroughs and extra quality points. | Yes No |
| IQU_ISI PC1.161.005_yoke part 3: Minimum requirement for steel: 1.97 T at 24 kA/m. | Yes No |
| IQU_ISI PC1.161.006_yoke part 4: Minimum requirement for steel: 1.97 T at 24 kA/m. | Yes No |
| IQU_ISI PC1.161.007_yoke part 5: Minimum requirement for steel: 1.97 T at 24 kA/m. | Yes No |
| IQU_ISI PC1.161.008_inner yoke part 5: Minimum requirement for steel: 1.97 T at 24 kA/m. | Yes No |
| Blank low carbon steel material for in house machining: The offer has to include following raw material: |  |
| IQU_ISI PC1.127.001_BLANK: Minimum requirement for steel: 1.97 T at 24 kA/m. | Yes No 4 pieces |
| IQU_ISI PC1.127.005_BLANK: Minimum requirement for steel: 1.97 T at 24 kA/m. | Yes No 2 pieces |
| IQU_ISI PC1.142.001_BLANK: Minimum requirement for steel: 1.97 T at 24 kA/m. | Yes No 3 pieces |
| IQU_ISI PC1.150.001_BLANK: Minimum requirement for steel: 1.97 T at 24 kA/m. | Yes No 2 pieces |
| IQU_ISI PC1.152.001_BLANK: Minimum requirement for steel: 1.97 T at 24 kA/m. | Yes No 3 pieces |
| Certificate: |  |
| The offer has to include data showing the experimental BH curve of offered low carbon steel: especially showing the value at H ≈ 24 kA/m (preferably up to 100 kA/m) | Yes No |
| Terminals: |  |
| The offer has to include the terminals for the cooling water and current as follows (Note: Swageloks excluded) - Cooling water terminals with the specified threads are included and soldered for Solenoid 1, Solenoid 2 and Solenoid 3. See drawing: IQU_ISI PC1.161.013_coupling coils - Current terminals (soldered) for Solenoid 1, Solenoid 2 and Solenoid 3 are included and soldered. See drawings: IQU_ISI PC1.161.011_current input coil coupling 1 and IQU_ISI PC1.161.012_current input coil coupling 2. | Yes No Yes No |
| Test assembly and factory test: |  |
| Test assembly has to be performed as is presented in IQU_ISI PC1.160.000b_solenoidsystem. Tube having outer diameter of 290 mm has to fit into the coil assembly as is presented in the drawing. NOTE: The test assembly (to test the adequate alignment of solenoids) can be realized with the axis of the solenoids in vertical orientation. The Customer will machine and send the supports for the test assembly (6 pieces: IQU-ISI PC1.161.010_middle block). | Yes No |
| Insulation test of Solenoid 1 and Solenoid 3: Conductor to ground (complete yoke) using 2 kV DC. Current leak less than 20 µA. Current leak test of Solenoid 2. Conductor to ground plate (both sides of Solenoid 2). | Yes No |
| Hydraulic leak tests for Solenoid 1, Solenoid 2 and Solenoid 3 using pressure of ≥ 12 bar (time ≥ 10 min). | Yes No |
| Water flow measurement at ΔP = 6 bar. The water flow (at 20° C) through each double wound double pancakes has to exceed 2 x 3.65 l/mm (7.3 l/mm). | Yes No |
| Electrical resistance at 20°C for Solenoid 1, Solenoid 2 and Solenoid 3: The measured resistance value Rmes has to confirm that the calculated electric power consumption P of Solenoid 1 and Solenoid 3 will not exceed 91 kW when the current value of I =1000 A is used (P = RmesI²). Solenoid 2 has to have identical resistance value/double wound double pancake. This requirement is met. | Yes No |
| Variant (optional): Different conductor as suggested can be used. |  |
| The supplier can suggest the use of different conductor. In this case the same total current (1000 A x 7 x 2 x 20 = 280 kA), or higher, has to flow through the cross section reserved for the pancakes (see drawings). In this case two requirements have to be met: 1) power consumption of injection and extraction solenoid will not exceed 91 kW (each) and 2) temperature rise ΔT of the cooling water will not exceed 30 K (when ΔP = 6 bar is used). The Customer has to accept the suggested conductor before the submission of Tender. | Yes No |
| Delivery time: |  |
| Maximum delivery time is 12 months as maximum (preferably less) | Yes No |