## Contractor Qualification Requirements

The Contractor shall demonstrate documented competence and certification in the following areas:

# Quality Management

- Valid certification according to EN ISO 9001 (Quality Management System).

# Welding and Structural Manufacturing

- Welding quality management certification in accordance with EN ISO 3834 (minimum ISO 3834-3, preferably ISO 3834-2).

- Qualified welding personnel certified in accordance with EN ISO 9606-1 (manual welding) and EN ISO 14732 (mechanized/automatic welding).

- Non-destructive testing (NDT) personnel certified in accordance with EN ISO 9712 in level 2.

- Compliance with EN 1090-1 (minimum EXC2) for load-bearing steel structures, where applicable.

- Demonstrated experience in structural verification of large welded assemblies in accordance with recognized European standards such as AD 2000 or EN 13445, including experience with third-party inspection or notified body involvement for components exceeding 3 meters in length.

# Vacuum System Experience

The Contractor shall demonstrate documented experience in:

- Manufacturing of large vacuum chambers and custom-made metallic flanges with lengths of at least 3 meters.

- Implementation of rotatable feedthrough systems operating between atmospheric pressure and vacuum conditions, with proven performance at vacuum levels of at least 10⁻⁶ mbar.

- High-vacuum applications including suitable surface treatment and corrosion protection of carbon steel components (e.g., passivation compatible with vacuum environments).

- Suppliers need to have personnel which is certificated according to EN 1779 to perform He-leak tests.

# Risk assessment

The Contractor shall demonstrate:

- Risk assessment in accordance with EN ISO 12100

# Safety-Related Control Systems

The Contractor shall demonstrate:

- Proven experience in the implementation of safety-related control systems in accordance with EN ISO 13849-1 (up to PL e) and/or IEC 61508 (up to SIL 3).

- Experience with fail-safe PLC systems (e.g., Siemens S7-1500F family or equivalent).

- Availability of qualified personnel for the programming, validation, and commissioning of safety-related PLC systems.

- Capability to ensure full compliance with the EU Machinery Regulation (EU) 2023/1230.

- Safety validation in accordance with EN ISO 13849-2 (SISTEMA project)

# Evidence of Experience

- The Contractor shall provide three reference projects demonstrating compliance with the above requirements. (Qualification requirements defined for this tender needs to be highlighted across these references.)

- Upon request, copies of valid certificates shall be submitted to the Company.