Specification: design and manufacturing of planar wave generators for PRIOR-II shock-wave experiments

1 – Overview

The Plasma Physics department at GSI and FAIR operates the PRIOR-II proton microscope for FAIR, which presently uses high-energy protons from the SIS-18 synchrotron to perform high-speed, high-resolution density measurements of dynamic processes occurring on the nanosecond range. An important part of the future experimental program with PRIOR-II within the framework of FAIR phase 0 are experiments on extreme pressures and dynamic material properties under high strain rates, which are planned to be achieved by high explosive driven shock-wave generators. This requires the development, characterization, and manufacturing of these generators.

The specification, divided into three subsections, covers the necessary simulation work to define the design, the subsequent experimental characterization regarding the planarity of the generated shock-wave and the function time, as well as the fabrication of the generators.

2 – Overview of the specifications

The scope of work for development and manufacturing includes the following:

- Simulation, design, fabrication, characterization, and certification of 22 planar shock-wave generators for experiments with the PRIOR-II proton microscope for FAIR. The total amount includes 14 generators for the planned experiments in 2028 and 8 for the characterization of two different variants by the manufacturer. An option for 6 additional generators for experiments should be quoted as well.

- A preliminary project schedule, including milestones and time constraints, should be provided with the offer. The manufacturing of the generators must be completed by the end of Q4/2027.

- Interim storage of the generators and detonators on the manufacturer’s premises until the experimental campaign takes place is necessary, as GSI and FAIR only have short-term storage facilities. The campaign is currently expected to be scheduled in 2028; we expect a maximum storage duration of 1 year (until the end of 2028).

- Approval/certification of the shock-wave generator by the BAM is required for transport of the devices from the manufacturer to FAIR. The transport shall be organized by the manufacturer.

- FAIR shall receive unrestricted rights of use to the final design of the shock-wave generator.

The offer should be divided into three separate subprojects as described below. The three subprojects include:

A. Design of the HE generator: hydrodynamic simulations

- The HE generator should be capable of launching a planar flyer plate with velocities up to several km/s. We expect a conically nested design composed of two different kinds of explosives; however, other suitable configurations are possible as well. The planarity of the flyer plate when impacting the target surface is defined as follows:

  - Within the field of view of the proton microscope (approx. 20 mm in diameter), the maximum time interval between the moment the center of the flyer strikes and the edge regions should be 100 ns.

  - The impact should be as perpendicular to the target surface as possible. The tilt should not exceed 5 mrad relative to the vertical axis. This is essential for a precise simulation of the experiment.

- Due to the limitations of the detonation chamber used, the maximum amount of explosives per generator must not exceed 250 g of TNT equivalent. In the preferred case of a combination of RDX/HMX, this results in a maximum amount of approximately 190 g, depending on the density or specific composition of the explosives used. To optimize the generator’s performance for a wide range of experiments that rely on generating different, maximum possible pressures within the material, we ask the manufacturer to utilize the limitations to the fullest extent possible (>85% of the maximum charge).

- While the field of view of the proton microscope is only about 20 mm, the flight plate itself should have a larger diameter to avoid any visible edge effects caused by the acceleration. A minimum diameter of 35–40 mm is desirable.

- The default configuration for optimizing the design to meet the requirements listed in 1–3 should be a copper flight plate with a thickness of approximately 2 mm.

- It is intended to vary the flyer plate velocity to adapt different experiments by selecting different materials and thicknesses. Only standard materials should be used for the flight plate:

  - Aluminum (alloy to be determined; no specification)

  - Copper

  - Tungsten (if necessary, WCu is possible due to its structural material properties)

Ideally, this should allow for flyer plate velocities of 1–4 km/s and generate pressures of 100 GPa+ in the experiment. Three configurations are sought, covering approximately 1 km/s, 2 km/s, and 4 km/s. Precise optimization of the material and/or the projectile thickness to achieve these velocities is not required (a deviation of +/- 25% is acceptable).

- A material with low mechanical impedance (foam) should be used between the high explosives and the flyer plate to ensure more uniform pressure transmission and to prevent spallation of the flyer plate.

- The general charge geometry as well as the geometry of the low-impedance material used between the explosive lens and the flight plate can be freely selected, provided the above requirements are met.

- Due to the timing limitations of the particle accelerator, the detonator for the generator must be an RP-2 EBW detonator from Teledyne. The incorporation of this type of detonator must be taken into account when designing the detonator lens.

B. Characterization of the HE generators

- A precise characterization of two different generator configurations with flyer plate velocities of 1 km/s and 2 km/s is required, focusing on the planarity of the flyer plate according to the above specifications and the total function time (and jitter of the function time between generators with identical configuration) from ignition to impact.

- The two different flyer plate velocities should be achieved by utilizing the simulation results from the previous phase of the project, employing various flight plate geometries and materials.

- Due to the geometry of the physics experiments and the available field of view, the propagation time of the shock-wave in the experiment itself is approximately 2–3 µs. The exposure window within this time period is about 0.8 µs. The measured jitter (uncertainty of the function time of identical generators) of the generators from the characterization should therefore ideally be within a range of no more than 250 ns (already including the uncertainty of the detonator itself).

- The generators must be tested with RP-2 detonators supplied by FAIR.

- FAIR provides a firing set for RP-2 detonators, which is compatible with the FAIR particle accelerator. This firing set must also be used for characterizing the generators.

- A precise measurement of the warping / bending of the flyer plate on impact is required.

- The methodology for determining the warping / bending of the flyer plate upon impact and for determining the function time is at the discretion of the manufacturer. When characterizing the generator, it must be taken into account that the experiments at FAIR are conducted under vacuum. A test at ambient pressure is expected to have a noticeable effect on the performance of the generator and on the timing due to the air cushion effect. A test in a vacuum chamber can be arranged; however, in this case, the appropriate diagnostic equipment must be brought on site by the manufacturer.

C. Manufacturing, storage (and provision) of the generators

- 14 shock-wave generators are required for the first experimental campaign at FAIR in the spring of 2027. The number of additional generators needed for preliminary characterization will be determined as the project progresses. We propose 4 generators per variant, 8 in total according to the characterization requirements.

- An option for an additional 6 generators should be provided.

- The mechanical design of the generator casing — and thus the interface with the experiments — will be finalized in collaboration with FAIR during the first two months of the project. A proposed concept for discussion is attached.

- The generator casing shall be provided by the manufacturer.

- FAIR will cover the costs and handle the necessary formalities associated with procuring and using Teledyne RP-2 EBW detonators for the described FAIR experiments and for the generator characterization. The manufacturer must provide the necessary data of the personal handling the detonators to fulfill the export control restrictions.

- FAIR itself has only short-term storage facilities for explosives. Accordingly, the detonators and completed generators must be stored on the manufacturers premises until the experiments are conducted. We expect the experiments to be scheduled in 2028; maximum storage time is until December 2028.

- The maximum storage duration for explosives at FAIR is a week (7 days). Therefore, the generators for the experiments must be supplied in two separate shipments to FAIR during the experiment period. An option for transport to FAIR should be part of the offer.

- If the manufacturer is within reasonable distance from FAIR (3 hr by car), FAIR personnel can pick up the generators if flexible pick-up can be provided. If this is not the case, an option for transportation according to the FAIR storage limitations must be provided by the manufacturer.

- Approval from the BAM is required for transport in Germany. FAIR submits the application for approval; to do so, the manufacturer must provide the BAM with data on the selected chemical formulation and the geometries.

3 – Acceptance criteria

- The criteria listed in 2 are target values that should be met, at a minimum, in simulations.

- The acceptance of the separate subprojects is defined as follows:

      - The FDR (final design report) includes a simulation and design report on the different generator configurations as specified, indicating that the criteria in 2 A can be met with the proposed design.

The report should furthermore contain a final mechanical design of the casing that has been developed together with FAIR. The report should indicate that the technical solutions chosen are suitable to fulfill the requirements from 2 B.

      - The FAT (factory acceptance test) includes the characterization of the developed generators either on the manufacturer’s premises or inside the FAIR blast chamber. A test report is required summarizing the performance of the generators according to the above specified criteria in 2 A and 2 B.

As part of the FAT, a report on the mechanical accuracy (measurement report) of all generators produced for the experiments is required.

The complete documentation (final simulation report, final test report, technical drawings, risk analysis, user manual) is due after manufacturing of the requested amount of generators.

4 – The offer must include the following information

- A preliminary description of the proposed concept in the form of sketches, technical drawings, and written explanations.

- A detailed project schedule in accordance with section 2, including the definition of payment milestones, is required.

- The specifications above are binding and must be met by the contractor.

5 – Appendix

Example of a generator housing (.step model). Different shapes/geometries are possible.

https://sf.gsi.de/f/5597282798c441dc9fed/